/* -*- mode: c++; coding: utf-8 -*- */

namespace Feel

{

/*! \page Notations Mathematical Concepts and Notations

<hr>

\tableofcontents

The mathematical concepts and the associated notations are defined on

this page and will be used throughout the Feel++ Online Documentation.

\section PolynomialNotations Polynomial Library

The polynomial library is composed of various bricks: \e (i) the

geometrical entities or convexes \e (ii) the prime basis in which we

express subsequently the polynomials, \e (iii) the definition and

construction of point sets in convexes (such as quadrature point sets)

and finally \e (iv) polynomials and finite elements.

\subsection Notations_Convexes Convexes

The supported convexes are simplices and hypercubes of topological dimension

\f$n\f$, \f$n=1,2,3\f$ lying in \f$\R^d\f$ such that \f$n \leqslant d

\leqslant 3\f$. The convexes are described geometrically in a standard way in

terms of their subentities (vertices, edges, faces, volumes), see for

example \cite MR1696933, and provide the ability to iterate over the entities

of a convex or of the same topological dimension inside a convex, e.g. iterate

over the edges of a tetrahedron.

\subsection Notations_PrimeBasis Prime basis: L^2 Orthonormal Polynomials

In order to express polynomials in the convexes defined previously, we

need to choose a \e prime \e basis, i.e., a basis in which all

polynomial families are expressed. Often, the choice falls on the

canonical basis (also known as the \e moment or \e monomial \e

basis). However, recent work by R.C. Kirby proposed to use the Dubiner

polynomials as a prime basis on the simplex. We extended these ideas

on the hypercubes using the Legendre polynomials. Other interesting

examples of prime basis being used are the Bernstein polynomials. Our

framework uses the Dubiner or Legendre basis as the default prime

basis. This choice simplifies the construction of finite elements due

to the hierarchical and \f$L^2\f$ orthogonality properties these basis

functions share. The choice of basis polynomials that are hierarchical

allows for an easy extraction of a basis spanning a subspace of the

polynomial space (which corresponds to extract a range of

coefficients), whereas \f$L^2\f$ orthogonality simplifies some

operations like numerical integration or the \f$L^2\f$ projection

(which is explicit in this case).  The use of these basis functions

proved to provide much better numerical stability, see \cite gpena.

Details on the construction of the Dubiner polynomials can be found

in \cite MR1696933 page 101. In practice, the prime basis is normalized.

\subsection Notations_psetconvex Point Sets on Convexes

Now we turn to the construction of point sets \f$\P\f$ defined on a convex \f$K\f$.

Point sets are represented algebraically by a matrix (rows are indexed by the

coordinates while columns are indexed by the points) and they are parametrized

by the associated convex and the numerical type. We recall that the convex is

decomposed in vertices, edges, faces, volumes. A similar decomposition is done

for the point sets: points are constructed and associated to their respective

entities on which they reside. This is crucial when considering continuous and

discontinuous Galerkin formulations.

The type of point sets supported are \e (i) the Equidistributed

point set, \e (ii) the Warpblend point sets on simplices see \cite warburton06,

\e (iii) Fekete points in simplices, see \cite MR1696933,

\e (iv) standard quadrature rules in simplices and finally

\e (v) Gauss, Gauss-Radau and Gauss-Lobatto and combinations in

simplices and hypercubes. It should be noted that the last family

is constructed from the computation of the zeros of the Legendre

polynomials on \f$[-1,1]\f$ including eventually the boundary vertices

\f$-1\f$, \f$1\f$ for the Radau and Lobatto flavors.

Warpblend and Fekete points are used with nodal basis on simplices which, when

constructed at these points, present much better interpolation properties (lower

Lebesgue constant, see \cite MR1696933). Note that the Gauss-Lobatto points are

the Fekete points in hypercubes.

\subsection Notatiopns_Pset Polynomial Set

After introducing in the previous sections the necessary bricks

to the construction of polynomials on simplices and hypercubes, we

now focus on the polynomial abstraction.

A polynomial set \f$\mathbb{P}\f$ is a template class parametrized by the prime

basis in which it is expressed and the field type in which it has its values:

scalar, vectorial or matricial. Its interface provides a number of operations

such as evaluation and derivation at a set of points, extraction of polynomials

or components (when the \c FieldType is \c Vectorial  or

\c Matricial) of a polynomial from a polynomial set .

One critical operation is the construction of the gradient of a polynomial (or a

polynomial set) expressed in the prime basis. This usually requires solving a

linear system where the matrix entries are given by the evaluation of the prime

basis and its derivatives at a set of points. Again the choice of set of points

is crucial here to avoid ill-conditioning and loss of accuracy. We choose

Gauss-Lobatto points for hypercubes and Warpblend or Fekete points for

simplicies as they provide a much better conditioning for the underlying system

matrix (a generalized Vandermonde matrix, see \cite gpena). % A trick that

% amounts for a appreciable gain in accuracy is to do these computations using

% higher precision types, e.g. \c dd_real, and then fall back in the end

% to the required numerical type, e.g. \c double.

\subsection Notations_fem Finite Elements and Other Polynomial Basis

\Feel supports modal basis, \eg Legendre or Dubiner, see

\cite MR1696933, \cite canuto_hussaini_quarteroni_zang_2, as well as finite elements

(FE) following the standard definition, set in \cite Ciarlet:2002:FEM:581834,

as a triplet \f$(K,\P,\Sigma)\f$ where \f$K\f$ is a convex, \f$\P\f$ the polynomial space

and \f$\Sigma\f$ the dual space. We describe now some features of the finite element

framework. The description of \f$K\f$ and \f$\P\f$ has been presented

previously and it remains to describe \f$\Sigma\f$. \f$\Sigma\f$ is a set of functionals

(which can be identified as degrees of freedom) defined in \f$\P\f$ with

values in \f$\R\f$, \f$\R^d\f$ or \f$\R^{d\times d}\f$. Several types of functionals can

then be instantiated which merely require basic operations like evaluation at a

set of points, derivation at a set of points, exact integration or numerical

integration. Some examples of functionals satisfying such requirements are

- evaluation at a point \f$x \in K\f$, \f$\ell_x : p \rightarrow p(x)\f$,

- derivation at a point \f$x \in K\f$ in the direction \f$i\f$, \f$\ell_{x,i} : p \rightarrow \frac{\partial p}{\partial x_i}(x)\f$,

-  moment integration associated with a polynomial \f$q \in \mathbb{P}(K)\f$, \f$\ell_q : p \rightarrow \int_{K} p q\f$.

A functional is represented algebraically by a vector whose entries result from

the application of the functional to the prime basis in which we express the

polynomials thanks to the bijection between \f$\calL(\P,\R)\f$ and

\f$\R^{\opdim(\P})\f$. Then applying the functional to a polynomial is just a scalar

product between the coefficient of this polynomial in the prime basis by the

vector representing the functional. For example the \e Lagrange \e element is

the finite element \f$(K, \mathbb{P}, \Sigma=\{\ell_{x_i}, x_i \in X \subset K\})\f$

such that \f$\ell_{x_i}( p_j ) = \delta_{ij}\f$ where \f$p_j\f$ is a Lagrange polynomial

and \f$X = \{x_i\}\f$ is a set of points defined in the convex \f$K\f$, for example the

Equidistributed, Warpblend or Fekete point sets. Other FE such as

\f$\mathbb{P}_{1,2}\f$-bubble, \f$\mathbb{R}\mathbb{T}_k\f$ or \f$\mathbb{N}_k\f$

polynomials are constructed likewise though they require a more involved

description.

\subsection Notations_Geometry Geometry

To conclude this section, one important object that is constructed with the help

of the polynomial library is the \e geometric \e transformation. Indeed all

polynomial set constructions are done on a reference convex, denoted \f$\hat{K}\f$,

and the geometrical transformation maps it to a convex in the physical space

which we denote \f$K\f$. This map, denoted \f$\tgeoK\f$, is the \f$C^1-\f$diffeomorphism

defined on \f$\hat{K} \subset \mathbb{R}^p, p=1,2,3\f$ such that the image is \f$K

\subset \mathbb{R}^d\f$, \ie \f$\tgeoK: \hat{K} \longrightarrow K\f$ for \f$p\leqslant d

\leqslant 3\f$. This map is contructed and associated to each convex \f$K\f$ in a

computational mesh \f$\calT_h\f$. Notice

that this last condition over \f$p\f$ and \f$d\f$ covers a large spectrum of geometrical

profiles. For instance, we handle lines or surfaces in \f$\mathbb{R}^3\f$.

The geometric transformation is constructed as a suitable linear combination of

Lagrange polynomials and therefore it can be a polynomial of arbitrary degree,

allowing thus meshes with elements that have curved edges/faces, see

\cite gpena_cprudhomme_acomen, \cite gpena_cprudhomme_aquarteroni. Another consequence

of \f$\tgeoK\f$ being a polynomial of a degree the user can choose, is the possibility

to define isoparametric (or subparametric or surparametric) finite elements, see

\cite gpena_cprudhomme_aquarteroni, \cite gpena. Lets denote \f$\kgeo\f$ the

polynomial order of the Lagrange basis in which \f$\tgeoK\f$ is expanded. If there

is no ambiguity, we keep the notation \f$\tgeoK\f$, otherwise we use the notation \f$\tgeoKkgeo\f$.

The class that implements the definition and evaluation of the

geometrical transformation also provides a function to evaluate its

gradient, automatic consequence of \f$\tgeoK\f$ being an element

belonging to a polynomial set. Another important transformation

associated with \f$\tgeoK\f$ is its inverse, \f$(\tgeoK)^{-1}\f$. In

the case of an affine transformation, the inverse is calculated

explicitely. However, if \f$\tgeoK\f$ is nonlinear, the

evaluation/differentiation of \f$(\tgeoK)^{-1}\f$ at a set of points

is performed with the help of a nonlinear solver (we have used the

nonlinear solver available in \c PETSc for these calculations.  The

inverse transformation plays an essential role in providing an

interpolation tool, all the advanced numerical methods use this tool

and hence the inverse geometrical transformation.

\section MeshNotations Mesh Notations

Let \f$\Omega\subset\R^d\f$, \f$d\ge 1\f$, denote a bounded connected domain.  We first

need to introduce a suitable discretization of \f$\Omega\f$, \f$\Omega_h \subset

\Omega\f$. Note that if \f$\Omega\f$ is a polyhedral domain then \f$\Omega_h = \Omega\f$.

We denote by \f$\Th\f$ a finite collection of nonempty, disjoint open simplices or

hypercubes \f$\Th=\{K = \tgeoK(\hatK)\}\f$ forming a partition of \f$\Omega_h\f$ such

that \f$h=\max_{K\in\Th} h_K\f$, with \f$h_K\f$ denoting the diameter of the element

\f$K\in\Th\f$.  We say that a hyperplanar closed subset \f$F\f$ of \f$\closure{\Omega}\f$ is

a mesh face if it has positive \f$(d{-}1)\f$-dimensional measure and if either there

exist \f$K_1,\,K_2\in\Th\f$ such that \f$F = \partial K_1\cap\partial K_2\f$ (and \f$F\f$ is

called an \emph{internal face}) or there exists \f$K\in\Th\f$ such that \f$F

= \partial K\cap\partial\Omega_h\f$ (and \f$F\f$ is called a \emph{boundary face}).

Internal faces are collected in the set \f$\Fhi\f$, boundary faces in \f$\Fhb\f$ and we

let \f$\Fh\eqbydef\Fhi\cup\Fhb\f$.  For all \f$F\in\Fh\f$, we define

\f$\calT_F\eqbydef\{K\in\Th\ST F\subset\partial K\}.\f$ For every interface

\f$F\in\Fhi\f$ we introduce two associated normals to the elements in \f$\calT_F\f$ and

we have \f$\normal_{K_1,F}=-\normal_{K_2,F}\f$, where \f$\normal_{K_i,F}\f$,

\f$i\in\{1,2\}\f$, denotes the unit normal to \f$F\f$ pointing out of \f$K_i\in\calT_F\f$.

On a boundary face \f$F\in\Fhb\f$, \f$\normal_F=\normal_{K,F}\f$ denotes the unit normal

pointing out of \f$\Omega_h\f$.

We also introduce

- the set of boundary elements

\f[

\calT^b_h=\{ K \in \calT_h\, \ST\, \partial K  \cap \partial \Omega \neq \emptyset\}

\f],

- the set of internal elements

\f[

\calT^i_h=\calT_h \backslash \calT^b_h

\f],

-  the set \f$\calN_h\f$ which collects the nodes of the mesh,

-  when \f$d=3\f$, \f$\calE_h\f$ which collects the edges of the mesh.

The collections \f$\calT_h, \calF_h, \calE_h, \calN_h\f$, as well as

the internal and boundary collections, are provided by our mesh data

structure and stored using the the Boost.Multi_index library

http://www.boost.org/libs/multi_index/doc/index.html.  The mesh

entities (elements, faces, edges, nodes) are indexed either by their

ids, the process id (i.e. the id given by MPI in a parallel context,

by default the current process id) to which they belong, their markers

(material properties, boundary ids...) or their location (whether the

entity is internal or lies on the boundary of the domain). Other

indices could certainly be defined, however those previous four

already allow a wide range of applications. Thanks to

Boost.Multi_index, it is trivial to retrieve pairs of iterators over

the entity's containers depending on the usage context. The pairs of

iterators are then turned into a range, see Boost.Range

http://www.boost.org/libs/range/index.html, to be manipulated by

integration, \ref Integrals, and projection, tools.

*/

}